Workflow

Coding requirement (make sure each step can be reproducable) (all codes in R, qmd file, each step has code chunk with clear comments, and markdown explanation, discussion, and layout) (create and save the qmd file)

1. Create fishnet based on Calgary city boundary shp file, set the grid size as 30 m. Make sure to keep the same crs. Choose crs as the local crs for Calgary. Each polygon has its own identification number (id).

2. Use inundation.tif to generate binary target variables. The raster is 10m resolution, so each grid will have around 9 cells. 0=no inundation and 1=inundation, other is nodata. Calculate the percentage of inundation cells in gird (fishnet), exceeding (>=) 0.3 will be 1 (true, inundation), < 0.3 will be 0 (false, no-inundation).

3. Use each fishnet (grid) to calculate the features on the list. The data are on data folder.

| Type | Feature | Data Source |
| --- | --- | --- |
| Topography | Mean elevation | calgary_dem.tif |
|  | Min. elevation | calgary_dem.tif |
|  | Elevation range | calgary_dem.tif |
|  | Mean slope | calgary_dem.tif |
|  | Max slope | calgary_dem.tif |
|  | Std. of elevation | calgary_dem.tif |
| Hydrology | Dist. to nearest stream | calgary_waterways.shp |
|  | Water cover area | calgary_surf_water_tif |
|  | River Density | calgary_waterways.shp |
| Water Shed Feature | Max flow accumulation | calgary_dem.tif |
|  | Upstream watershed area of fishnet centroid point | calgary_dem.tif |
| Land cover | Impervious surface ratio | calgary_Imp_surf.geojson |
|  | Vegetation cover proportion | calgary_landcover.geojson |
|  | Open soil land proportion | calgary_landcover.geojson |
|  | Agricultural land proportion | calgary_landcover.geojson |
| Built environment | Building cover ratio | calgary_building.shp |

The final output will be the gdf (geodataframe) of fishnet with columns of target variable and features and id. Use this to do later Logistic model.

3.1 The details of each feature calculations are follows:

| Mean elevation |  |
| --- | --- |
| Min. elevation |  |
| Elevation range |  |
| Mean slope |  |
| Max slope |  |
| Std. of elevation |  |
| Dist. to nearest stream |  |
| Water cover area |  |
| River Density |  |
| Max flow accumulation |  |
| Upstream watershed area of fishnet centroid point |  |
| Impervious surface ratio |  |
| Vegetation cover proportion |  |
| Open soil land proportion |  |
| Agricultural land proportion |  |
| Building cover ratio |  |

4 Exploratory Data Analysis and Data cleaning (All plotting and mapping should use ggplot). Outputs are: 4.1 The histogram and spatial distribution map of target variables and features. 4.2 Use VIF test and correlation Matrix to test the multicollinearity of features. 4.3 Do rational, realistic, and minior data cleaning to make model robust for validation and predicting another city.

5 Build the (gml) Logistic model. Do z-score normalization for all features before modeling.

6 Validation and Diagnostics.

This should contain your final logistic regression model summary table, your ROC curve plot, and a table of confusion metrics (True Positives, False Positives, etc). Please discuss the modeling process, the training and testing process, comment on the accuracy of the model, and make a table that uses simple statements to describe the four model outcomes ("A True Positive is...").

Output: final logistic regression model summary table, your ROC curve plot, and a table of confusion metrics (True Positives, False Positives, etc)